APNIC Training (PACNOG4)

Forensic Exercises – Team Cymru

- Look at the  two files icmp1.txt and icmp2.txt. These flows were generated from the file icmp.pcap. Can you explain the representation of the ICMP type 3 code 3 entry (127.0.0.1:0:771)?

- Given your explanation for question 3 how would a ICMP “Communication administratively prohibited by filtering” packet be represented? Hint:  determine what the values the code field in ICMP can have.

- Examine the  5 packet captures scan1.pcap – scan5.pcap in Wireshark. Describe in detail what you can determine is occurring here.

- Examine the file darknet.pcap. Analyze this file and find 10 interesting anomalies.

- Open flood1.pcap with Wireshark. Describe the type of traffic in the packet capture. How does the tool that generated this traffic perform a denial of service attack? How many packets per second would a botnet comprising of 500,000 hosts using this tool generate?

- Open flood2.pcap with Wireshark. Describe the type of traffic in the packet capture. How does the tool that generated this traffic perform a denial of service attack? How many packets per second would a botnet comprising of 500 hosts using this tool generate?

- Open flood3.pcap with Wireshark. Describe the type of traffic in the packet capture. How does the tool that generated this traffic perform a denial of service attack? How many packets per second would a botnet comprising of 1,000,000 hosts using this tool generate?

- Open query.pcap and amplification.pcap from the same folder. The traffic generated in these packet captures use an amplification technique, describe what it is. What amplification ratio does this technique give?

Control Channel Traffic

In the DDoS folder you can also find packet captures from various  types of botnet control channels.

- Open control1.pcap with Wireshark. This packet capture includes the control channel that goes with the DoS traffic found in flood1.pcap.

- Describe the control channel and how the infected host was instructed to DoS the target (www.e-gold.com).

- In this capture the DoS traffic can be read very clearly, describe the traffic paying attention to the exponential backoff and SYN frequency.

- Open control2.pcap with Wireshark. This packet capture contains a control channel from an infected host.

- Describe the control channel

- What frequency is the control channel being polled?

- Open control3.pcap with Wireshark. This packet capture includes the control channel that goes with the DoS traffic found in flood3.pcap.

- Describe the control channel

- How many hosts are being used to control the host?